# تکلیف 9 درس ریز پردازنده

آریا بنایی زاده 9431029

# 1.

## الف)

تراشه انتخاب شده، تراشه  IDT7134SAاست.

پارامتر های زمانی

## ب)

با توجه به شکل بالا مدار را میبندیم (همچنین فایل شبیه سازی در پیوست قرار دارد)

توجه شود که قطعه 4 کیلوبایتی در هیچ یک از کتابخانه های (در اینترنت و در خود پروتئوس یافت نشد) بنابراین در شکل بالا به جای sram های 4 کیلوبایتی از sram 2 کیلوبایتی استفاده نمودیم اما کلیت شکل با حالت مورد نظر تفاوتی ندارد صرفا اگر قطعا 4 کیلوبایتی را جایگزین کنیم، پایه های A11 را در امتداد پایه های قبل و به PD4 وصل میکنیم.

## ج)

با توجه به شکل دیکودر سه به هشت مناسب است

## د)

READ

; Read Data from Address 0700, SRAM with 20ns tAVQV (Address Access time), Result in R0

LDI R16, 0; Address: Low Byte

LDI R17, 7; Address: High Byte

CALL MemRead

MemRead:

LDI R18, FFH

OUT DDRC, R18 ; PORTC is Output

OUT DDRD, R18 ; PORTD is Output

LDI R18, 00H

OUT DDRA, R18 ; PORTA is Input

OUT PORTC, R16

ANDI R17, 3FH; SRAM #1 Enabled; decoder number should be zero

ORI R17, 0; Read Enabled

OUT PORTD, R17

NOP ; 1NOP=1Clock=62.5ns>tAVQV=20ns

IN R0, PINA

RET

WRITE

; Write Data to Address 1FFFH, SRAM with 20ns tWLWH (Write Pulse width), ; 12ns tDVWH (Data Valid To End of Write), and 0ns tWHDX (Data Hold Time), Data in R20

LDI R16, 0; Address: Low Byte

LDI R17, 40H; Address: High Byte ; 01000000 00000000 ;5th sram address

LDI R20, 60H

CALL MemWrite

MemWrite:

LDI R18, FFH

OUT DDRC, R18 ; PORTC is Output

OUT DDRD, R18 ; PORTD is Output

OUT DDRA, R18 ; PORTA is Output

OUT DDRB, R18 ; PORTB is Output

OUT PORTA, R20 ; data

OUT PORTC, R16

ANDI R17, 3FH; ; SRAM #1 Enabled; SRAM #2 Disabled,

ORI R17, A0H; ; Output Disabled, Write Pin=0; OUT PORTB, R17 ;A0

NOP ; 1NOP=1Clock=62.5ns> tWLWH=20ns

SBI PORTB, 0 ; Write Pin=1

NOP ; 1NO ;1Clock=62.5ns> tDVWH=12ns

RET

We به پورت b متصل شده است با توجه به شکل آدرس 4000 شکل در واقع آدرس صفر sram پنجم است بنابراین در برنامه تنظیمات را به این حالت قرار میدهیم(عدد دیکودر باید 4 شود)

# 2.

حافظه ی برنامه این خانواده عموما از جنس فلش و حافظه رم آن ها SRAM می باشد.

در جدول زیر حجم حافظه ها را بر اساس هر کدام از این ریزپردازنده ها می بینیم.

Mouser Part #; Series; Core; Program Memory Size; Data RAM Size. 511-STM32F303VBT6TR; STM32F3; ARM Cortex M4; 128 kB; 32 kB. 511-STM32F051C8T6TR; STM32F0; ARM Cortex M0; 64 kB; 8 kB. 511-STM32F301C8T6; STM32F3; ARM Cortex M4; 64 kB; 16 kB. 511-STM32F303RBT7; STM32F3; ARM Cortex M4; 128 kB; 32 kB. 511-STM32F439BIT6; STM32F4; ARM Cortex M4; 2 MB; 256 kB. 511-STM32F437ZIT7; STM32F4; ARM Cortex M4; 2 MB; 256 kB. 511-STM32F301R6T6; STM32F3; ARM Cortex M4; 32 kB; 16 kB. 511-STM32F765VGT7; STM32F7; ARM Cortex M7; 1 MB; 512 kB. 511-STM32F215VGT7; STM32F2; ARM Cortex M3; 1 MB; 128 kB. 511-STM32F070C6T6TR; STM32F0; ARM Cortex M0; 32 kB; 6 kB. 511-STM32F103ZEH7TR; STM32F1; ARM Cortex M3; 512 kB; 64 kB. 511-STM32F071CBT7TR; STM32F0; ARM Cortex M0; 128 kB; 16 kB. 511-STM32F100RDT7B; STM32F1; ARM Cortex M3; 384 kB; 32 kB. 511-STM32F745VEH6TR; STM32F7; ARM Cortex M7; 512 kB; 320 kB. 511-STM32F437AIH6TR; STM32F4; ARM Cortex M4; 2 MB; 256 kB.